# MILTECH / Techaya Switch — DHCP Troubleshooting Field Guide

Offline reference — ping works, DHCP doesn't

## 0. IMMEDIATE FALLBACK — Get devices talking right now, no DHCP needed

Since ping already works, IP routing between your devices is fine. You can bypass DHCP entirely by assigning static IPs manually.

Step 1 — Find a valid address range. On a device that can currently ping others, run one of:

- Windows: ipconfig /all and route print

- Linux/Mac: ip addr and ip route (or route -n)

Note the subnet (e.g. 192.168.105.x /24) and the default gateway address.

Step 2 — Assign static IPs to the devices that fail DHCP. Pick an unused address in the same subnet as the gateway you found, and set the same subnet mask and gateway.

- Windows: Control Panel → Network → Adapter Properties → IPv4 → "Use the following IP address." Enter IP, subnet mask, gateway. DNS can be left blank or set to the gateway.

- Windows (CLI, faster):

netsh interface ip set address "Ethernet" static 192.168.105.50 255.255.255.0 192.168.105.1

- Linux:

sudo ip addr add 192.168.105.50/24 dev eth0

sudo ip route add default via 192.168.105.1

Give every device that needs to talk to each other an address in the same subnet/VLAN if possible. This sidesteps the DHCP/VLAN issue completely and is the most reliable option if you're short on time.

## 1. Identify your exact model

Check the chassis label/engraving. The MILTECH family varies significantly by model:

| Model | Type | Notes |
| --- | --- | --- |
| MILTECH 304 / 308 | Basic switch, 8x 10/100 | Limited/no IP management on some variants |
| MILTECH 908 | 8-port managed L2 | DHCP relay/option 82 is an optional feature, only on part number 1-908-101 |
| MILTECH 9012 / 9012C / 9012C/G | 12-port managed L2/L3 switch + internal SBC running Linux + Cisco IOS router | Full router-on-a-stick topology, see below |
| MILTECH 9022 | 12x copper + 4x fiber, L3 managed | Static/RIPv1/v2/OSPF routing |
| MILTECH 9028 | 12-port, IP67 | Managed L2/L3 |

If you can't find a label, the default management IP 192.168.1.111 (mask 255.255.255.0) is common across most managed variants — try that first.

## 2. Default management access

| Access method | Setting |
| --- | --- |
| Web / Telnet | 192.168.1.111 / 255.255.255.0 (switch management) |
| Serial (switch side) | Connect to physical Port #1, RS-232, 115200 baud, 8 data bits, No parity, 1 stop bit, No flow control |
| Switch login | Username: admin, Password: (blank — just press Enter) |
| Secondary switch IP (9012C/G only, for SBC reachability) | 192.168.101.254 / 255.255.255.0 |
| SBC (Linux) login (9012C/G only) | Username: root, Password: 123456 |
| Cisco router CLI access (9012C/G only) | From Linux prompt type cisco, press Enter, then enable, password cisco |

If admin with a blank password doesn't work, the unit may have been previously configured with a custom password — in that case the static-IP fallback in Section 0 is your best bet without a factory reset.

## 3. Root cause: why ping works but DHCP doesn't (VLAN-per-port design)

On the managed 9012-family units, each physical port ships on its own separate VLAN by default:

Port 1  = VLAN 1   (gateway 192.168.101.1)

Port 2  = VLAN 2   (gateway 192.168.102.1)

Port 3  = VLAN 3   (gateway 192.168.103.1)

...

Port 11 = VLAN 11  (gateway 192.168.111.1)

Port 12 = Trunk to internal router/SBC (carries all VLANs)

An internal router (Cisco IOS on the SBC) routes unicast traffic between these VLANs — which is why ping across ports/VLANs works. But DHCP relies on broadcast traffic, which does not cross VLAN boundaries by itself. If your DHCP server is on a different port/VLAN than the client that's failing, the DHCP broadcast simply never reaches it — even though the router would happily route a ping between them.

Fix, if you have management access: configure a DHCP relay (Cisco term: ip helper-address) on the router subinterface for the client's VLAN, pointing at the DHCP server's real IP address:

interface Ethernet0/0.3          ! example: VLAN 3 (Port 3)

ip helper-address 192.168.105.10  ! real IP of your DHCP server

Do this for every VLAN/port that needs DHCP from a server sitting on a different port.

Fix, if you don't have time/access to configure this: physically move the DHCP server and the affected client onto the same port group/VLAN, or just use the static-IP fallback in Section 0.

## 4. Show / verification commands

### A. Cisco IOS router side (9012C/G — access via cisco then enable as in Section 2)

These are standard Cisco IOS commands, confirmed applicable since the embedded router runs Cisco IOS 15.6(3)M1 (5921 ESR):

| Command | What it tells you |
| --- | --- |
| show ip interface brief | Lists all VLAN subinterfaces (Eth0/0.1–0/0.11), their IPs, and up/down status |
| show running-config | Full config — look for ip helper-address lines under each subinterface |
| show ip interface Ethernet0/0.X | For one specific VLAN subinterface — explicitly shows "Helper address is ..." if a relay is configured, plus IP/mask |
| show ip route | Confirms the router has a route to every VLAN subnet (101.0–111.0/24) |
| show arp | Confirms whether the router has learned the MAC/IP of devices on each VLAN — if a device never appears here, its broadcasts/traffic aren't reaching the router at all |
| show interfaces Ethernet0/0 | Status/error counters on the trunk parent interface (port #12) |
| show version | Confirms IOS version and uptime, useful if you suspect it crashed/reloaded |

Fastest single check: show running-config | include helper — if this comes back empty, no DHCP relay is configured anywhere, which on its own would explain the symptom if your server and client are on different ports.

### B. From any connected PC (works no matter what switch model you have)

| OS | Command | What it tells you |
| --- | --- | --- |
| Windows | ipconfig /all | Current IP, whether it's a real lease or 169.254.x.x self-assigned |
| Windows | arp -a | Devices the PC can see at Layer 2 — helps confirm what's reachable |
| Windows | tracert <gateway-ip> | Whether traffic is routed (multiple hops) or local (one hop) |
| Linux/Mac | ip addr / ip route | Same as above for IP/gateway |
| Linux/Mac | ip neigh (or arp -a) | Same as Windows ARP table check |

### C. The switch-chip side (ports 1–12, separate from the Cisco router)

Be aware: this portion runs its own proprietary management software (referenced in the manual as a separate "Ethernet-Switch CLI user guide"), and the exact command syntax for that CLI isn't something I have reliable documentation for — guessing at exact syntax here could cost you time rather than save it. Two safer options:

- Use the Web GUI instead (192.168.1.111 per Section 2) — look for a VLAN membership/port table and a DHCP snooping/relay page; this is the most universal way to check port-to-VLAN assignment without needing exact CLI syntax.

- If you do get into a CLI/serial prompt on the switch side, type ? or help at the prompt — nearly every network CLI supports this to list valid commands, and it'll give you the actual syntax for that firmware rather than a guess.

## 5. Quick diagnostic checklist

- Confirm it's really DHCP failing. Check the device's address — 169.254.x.x (Windows/most OSes) confirms DHCP timed out and the device self-assigned. No address at all, or an old static one, means something different is going on.

- Check which physical ports the DHCP server and the failing client are plugged into.

- If on different ports → almost certainly the VLAN/relay issue above.

- If on the same port/VLAN and still failing, check for:

  - DHCP snooping enabled with the server's port marked "untrusted"

  - Spanning-tree port stuck in blocking/listening state (wait 30-60s after plugging in, or check port status)

  - Broadcast storm control throttling DHCP broadcasts

  - A port administratively shut down in the switch config

  - An ACL or firewall rule blocking UDP ports 67/68

- Hardware-level (no config access needed):

  - Check the per-port Link/Activity LED — solid/flashing = physical link is up

  - Swap the cable and/or try a different port

  - Power-cycle in order: DHCP server device → switch → end devices, ~30 sec between each

## 6. Hard reset (last resort, 9012C/G only)

There's a physical reset function on Port #3:

- Momentary contact between Reset pin (M) and GND (L): graceful shutdown of the whole unit (switch + SBC). A second momentary contact powers it back up.

- Holding for over 3 seconds: forces a hard shutdown of switch and SBC. You'll need to disconnect and reconnect power afterward to bring it back up.

This does not reset configuration to factory defaults by itself — it's a power-cycle mechanism, not a config wipe. Use it if the unit is unresponsive, not as a way to clear settings.

## Summary — fastest path under time pressure

- Try static IPs (Section 0) to get connectivity working immediately, regardless of root cause.

- If you have a few minutes, log into management (Section 2), and run show running-config | include helper and show ip interface brief (Section 4) to confirm the VLAN/relay theory (Section 3) in seconds.

- If access fails or there's no time, keep DHCP server and critical clients on the same physical port/VLAN group as a structural workaround.