Nexus vPC — Full Configuration v2

Nexus 1 + Nexus 2  |  Exact Port Assignments from Your Document  |  Fortigate + Servers + PA

Netmagic Datacenter  | Hassan  | Draft — Verify before applying

CRITICAL  DRAFT — All port numbers taken directly from your port assignment document. Verify physical cabling before applying.

# Port Assignment Reference

- ! Topology layer summary:

- ! [SERVER1]          [SERVER2]

- !     |                  |

- !  vPC trunk          vPC trunk

- !     |                  |

- ! [NEXUS 1] <--vPC--> [NEXUS 2]   (Keepalive: 10.10.10.1/2)

- ! LAN: 172.27.8.0/24

- !     |                  |

- !  FW LACP + FS MCLAG

- !     |                  |

- ! [FGCORE1]         [FGCORE2]     (Fortigate Core L3)

- !     |                  |

- !  FW LACP + FS MCLAG

- ! LAN: 172.27.8.0/24

- !     |                  |

- ! [FGSW1]           [FGSW2]       (Fortigate SW)

- !     |                  |

- !  FS MCLAG + PA LACP

- !     |                  |

- ! [PA PARAM 1] [PA PARAM 2] (Palo Alto)

TEST

# Port Assignment Plan

ℹ  Adjust these port numbers to match your actual Nexus hardware slot/port layout.

! ============================================================

! PORT ASSIGNMENT — NEXUS 1 & NEXUS 2 (symmetric)

! ============================================================

! Ethernet1/49-50 	= vPC Peer-Link (ISL between Nexus1 <-> Nexus2 Po50)

! Ethernet1/47-48 	= vPC Keep-alive (Po100, 10.10.10.1/2 Po 48)

! Ethernet1/46 		= Fortigate Core1 (Po46 on Nexus1 / Po46 on Nexus2)

! Ethernet1/45	 	= Fortigate Core2 (Po45 on Nexus1 / Po45 on Nexus2)

! Ethernet1/4-5 		= SERVER SW Po2 dual-home (Po104 on Nexus1 / Po104 on Nexus2)
! Ethernet1/6-7		= SERVER SW Po3 dual-home (Po105 on Nexus1 / Po105 on Nexus2)
! Ethernet1/8-9		= SERVER SW Po4 dual-home (Po106 on Nexus1 / Po106 on Nexus2)
! Ethernet1/10-11 	= SERVER SW Po5 dual-home (Po107 on Nexus1 / Po107 on Nexus2)
! Ethernet1/12-13 	= SERVER SW Po6 dual-home (Po108 on Nexus1 / Po108 on Nexus2)
! Ethernet1/14-15 	= SERVER SW Po7 dual-home (Po109 on Nexus1 / Po109 on Nexus2)
! Ethernet1/16-17	= SERVER SW Po8 dual-home (Po110 on Nexus1 / Po110 on Nexus2)
! Ethernet1/18-19 	= SERVER SW Po9 dual-home (Po111 on Nexus1 / Po111 on Nexus2)
! Ethernet1/20-21 	= SERVER SW Po10 dual-home (Po112 on Nexus1 / Po112 on Nexus2)
! Ethernet1/22-23 	= SERVER SW Po11 dual-home (Po113 on Nexus1 / Po113 on Nexus2)

! Loopback0 		= NTP/SNMP source

! Loopback1 		= 172.27.7.1/30 (replicated from CBS)

# Section 1 — Feature Enables (Both Switches — Apply First)

INFO  Enable features BEFORE any interface or vPC config. 'feature vpc' must be present before configuring vpc domain.

## NEXUS 1

- hostname Netmagic-Nexus1

- feature lacp

- feature vpc

- feature interface-vlan

- feature lldp

- feature ssh

- feature snmp

- feature hsrp

- ip routing

- no feature telnet

## NEXUS 2

- hostname Netmagic-Nexus2

- feature lacp

- feature vpc

- feature interface-vlan

- feature lldp

- feature ssh

- feature snmp

- feature hsrp

- ip routing

- no feature telnet

# Section 2 — vPC Domain, Dual Keepalive & Peer-Link (Po50)

CRITICAL  Keepalive links (E1/47 + E1/48) MUST be UP and both IPs reachable BEFORE Po50 peer-link comes up. Bring up keepalives first, verify reachability, then bring up Po50.

INFO  Dual keepalive: primary 10.10.10.x on E1/47, secondary 20.20.20.x on E1/48. NX-OS uses primary by default. Secondary is fallback.

WARN  Mask corrected to /30 (255.255.255.252) on both sides — your document showed /24 on Nexus2 side which would overlap with other subnets.

## NEXUS 1 — Keepalive Links (E1/47, E1/48)

interface Ethernet1/47

description vPC-Keepalive-PRIMARY-10.10.10.x

no switchport

ip address 10.10.10.1 255.255.255.252

no shutdown

interface Ethernet1/48

description vPC-Keepalive-SECONDARY-20.20.20.x

no switchport

ip address 20.20.20.1 255.255.255.252

no shutdown

## NEXUS 1 — vPC Domain 10

vpc domain 10

peer-keepalive destination 10.10.10.2 source 10.10.10.1

peer-keepalive destination 20.20.20.2 source 20.20.20.1

peer-gateway

auto-recovery

auto-recovery reload-delay 360

delay restore 150

ip arp synchronize

## NEXUS 1 — Peer-Link Po50 (E1/49 + E1/50)

interface port-channel50

description vPC-Peer-Link-ISL-to-Nexus2

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

spanning-tree port type network

vpc peer-link

no shutdown

interface Ethernet1/49

description vPC-ISL-to-Nexus2-MemberA

switchport

switchport mode trunk

channel-group 50 mode active

no shutdown

interface Ethernet1/50

description vPC-ISL-to-Nexus2-MemberB

switchport

switchport mode trunk

channel-group 50 mode active

no shutdown

## NEXUS 2 — Keepalive Links (E1/47, E1/48)

interface Ethernet1/47

description vPC-Keepalive-PRIMARY-to-Nexus1

no switchport

ip address 10.10.10.2 255.255.255.252

no shutdown

interface Ethernet1/48

description vPC-Keepalive-SECONDARY-to-Nexus1

no switchport

ip address 20.20.20.2 255.255.255.252

no shutdown

## NEXUS 2 — vPC Domain 10

vpc domain 10

peer-keepalive destination 10.10.10.1 source 10.10.10.2

peer-keepalive destination 20.20.20.1 source 20.20.20.2

peer-gateway

auto-recovery

auto-recovery reload-delay 360

delay restore 150

ip arp synchronize

## NEXUS 2 — Peer-Link Po50 (E1/49 + E1/50)

interface port-channel50

description vPC-Peer-Link-ISL-to-Nexus1

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

spanning-tree port type network

vpc peer-link

no shutdown

interface Ethernet1/49

description vPC-ISL-to-Nexus1-MemberA

switchport

switchport mode trunk

channel-group 50 mode active

no shutdown

interface Ethernet1/50

description vPC-ISL-to-Nexus1-MemberB

switchport

switchport mode trunk

channel-group 50 mode active

no shutdown

# Section 3 — VLAN Database (Identical on Both Switches)

- ! ============================================================

- ! VLAN DATABASE

- ! ============================================================

- vlan 11

- name BASE-VLAN

- vlan 12

- name DATABASE-VLAN

- vlan 13

- name ANCILLARY-VLAN

- vlan 14

- name HSM-VLAN

- vlan 15

- name PMP-VLAN

- vlan 16

- name NAS-VLAN

- vlan 20

- name ATM-VLAN

- vlan 21

- name CARD-MGMT-VLAN

- vlan 22

- name ECOMMERCE-VLAN

- vlan 23

- name WEB-SERVERS-VLAN

- vlan 24

- name IMPS-SWITCH

- vlan 25

- name IMPS-WEB

- vlan 26

- name IMPS-EXTERNAL

- vlan 27

- name UPI-SWITCH

- vlan 28

- name UPI-WEB

- vlan 29

- name UPI-EXTERNAL

- vlan 30

- name BBPS-SWITCH

- vlan 31

- name BBPS-WEB

- vlan 32

- name BBPS-INTERNAL

- vlan 33

- name FUTURE-USE

- vlan 34

- name AEPS-EKYC

- vlan 35

- name WEB-EKYC

- vlan 36

- name CBS

- vlan 37

- name BBPS-INTERNAL-2

- vlan 38

- name FUTURE-USE-2

- vlan 39

- name UAT-OOB

- vlan 40

- name UAT

- vlan 41

- name PAPG

- vlan 45

- name L2-ONLY

# Section 4 — Spanning Tree

CRITICAL  Set STP priority BEFORE any trunk comes up. Nexus 1 = primary root (4096), Nexus 2 = secondary root (8192).

## NEXUS 1 — STP Primary Root

spanning-tree mode rapid-pvst

spanning-tree vlan 1-39,45 root primary

## NEXUS 2 — STP Secondary Root

spanning-tree mode rapid-pvst

spanning-tree vlan 1-39,45 root secondary

# Section 5 — Fortigate Core 1 & 2 (E1/46 → Po46, E1/45 → Po45)

INFO  Single physical port per Nexus per Fortigate (E1/46 for FGCore1, E1/45 for FGCore2). vPC provides redundancy across the two Nexus switches. If you need per-switch port-channel resilience, add a second port per Fortigate and bundle.

## Fortigate Core 1 — Po46, vPC 46

! --- NEXUS 1 ---

interface port-channel46

description FGCore1-vPC46

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 46

no shutdown

interface Ethernet1/46

description FGCore1-Link-Nexus1

switchport

switchport mode trunk

channel-group 46 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel46

description FGCore1-vPC46

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 46

no shutdown

interface Ethernet1/46

description FGCore1-Link-Nexus2

switchport

switchport mode trunk

channel-group 46 mode active

no shutdown

## Fortigate Core 2 — Po45, vPC 45

! --- NEXUS 1 ---

interface port-channel45

description FGCore2-vPC45

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 45

no shutdown

interface Ethernet1/45

description FGCore2-Link-Nexus1

switchport

switchport mode trunk

channel-group 45 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel45

description FGCore2-vPC45

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 45

no shutdown

interface Ethernet1/45

description FGCore2-Link-Nexus2

switchport

switchport mode trunk

channel-group 45 mode active

no shutdown

# Section 6 — Server Port-Channels Po104–Po113 (E1/4–E1/23)

INFO  Each server is dual-homed across Nexus 1 and Nexus 2. The same port-channel number and vPC number are used on both switches. Po110 and Po111 use lacp rate fast (was 'lacp timeout short' on CBS for FSNETNTX1127/1128).

WARN  Server-side NIC bonding must be LACP (mode 4 / 802.3ad on Linux, LACP on VMware ESXi) before cutover. Verify before connecting.

## FSNETNTX1121 — Po104, vPC 104  (Ethernet1/4 + Ethernet1/5)

! --- NEXUS 1 ---

interface port-channel104

description FSNETNTX1121-vPC104

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 104

no shutdown

interface Ethernet1/4

description FSNETNTX1121-Link1-Nexus1

switchport

switchport mode trunk

channel-group 104 mode active

no shutdown

interface Ethernet1/5

description FSNETNTX1121-Link2-Nexus1

switchport

switchport mode trunk

channel-group 104 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel104

description FSNETNTX1121-vPC104

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 104

no shutdown

interface Ethernet1/4

description FSNETNTX1121-Link1-Nexus2

switchport

switchport mode trunk

channel-group 104 mode active

no shutdown

interface Ethernet1/5

description FSNETNTX1121-Link2-Nexus2

switchport

switchport mode trunk

channel-group 104 mode active

no shutdown

## FSNETNTX1122 — Po105, vPC 105  (Ethernet1/6 + Ethernet1/7)

! --- NEXUS 1 ---

interface port-channel105

description FSNETNTX1122-vPC105

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 105

no shutdown

interface Ethernet1/6

description FSNETNTX1122-Link1-Nexus1

switchport

switchport mode trunk

channel-group 105 mode active

no shutdown

interface Ethernet1/7

description FSNETNTX1122-Link2-Nexus1

switchport

switchport mode trunk

channel-group 105 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel105

description FSNETNTX1122-vPC105

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 105

no shutdown

interface Ethernet1/6

description FSNETNTX1122-Link1-Nexus2

switchport

switchport mode trunk

channel-group 105 mode active

no shutdown

interface Ethernet1/7

description FSNETNTX1122-Link2-Nexus2

switchport

switchport mode trunk

channel-group 105 mode active

no shutdown

## FSNETNTX1123 — Po106, vPC 106  (Ethernet1/8 + Ethernet1/9)

! --- NEXUS 1 ---

interface port-channel106

description FSNETNTX1123-vPC106

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 106

no shutdown

interface Ethernet1/8

description FSNETNTX1123-Link1-Nexus1

switchport

switchport mode trunk

channel-group 106 mode active

no shutdown

interface Ethernet1/9

description FSNETNTX1123-Link2-Nexus1

switchport

switchport mode trunk

channel-group 106 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel106

description FSNETNTX1123-vPC106

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 106

no shutdown

interface Ethernet1/8

description FSNETNTX1123-Link1-Nexus2

switchport

switchport mode trunk

channel-group 106 mode active

no shutdown

interface Ethernet1/9

description FSNETNTX1123-Link2-Nexus2

switchport

switchport mode trunk

channel-group 106 mode active

no shutdown

## FSNETNTX1124 — Po107, vPC 107  (Ethernet1/10 + Ethernet1/11)

! --- NEXUS 1 ---

interface port-channel107

description FSNETNTX1124-vPC107

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 107

no shutdown

interface Ethernet1/10

description FSNETNTX1124-Link1-Nexus1

switchport

switchport mode trunk

channel-group 107 mode active

no shutdown

interface Ethernet1/11

description FSNETNTX1124-Link2-Nexus1

switchport

switchport mode trunk

channel-group 107 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel107

description FSNETNTX1124-vPC107

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 107

no shutdown

interface Ethernet1/10

description FSNETNTX1124-Link1-Nexus2

switchport

switchport mode trunk

channel-group 107 mode active

no shutdown

interface Ethernet1/11

description FSNETNTX1124-Link2-Nexus2

switchport

switchport mode trunk

channel-group 107 mode active

no shutdown

## FSNETNTX1125 — Po108, vPC 108  (Ethernet1/12 + Ethernet1/13)

! --- NEXUS 1 ---

interface port-channel108

description FSNETNTX1125-vPC108

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 108

no shutdown

interface Ethernet1/12

description FSNETNTX1125-Link1-Nexus1

switchport

switchport mode trunk

channel-group 108 mode active

no shutdown

interface Ethernet1/13

description FSNETNTX1125-Link2-Nexus1

switchport

switchport mode trunk

channel-group 108 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel108

description FSNETNTX1125-vPC108

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 108

no shutdown

interface Ethernet1/12

description FSNETNTX1125-Link1-Nexus2

switchport

switchport mode trunk

channel-group 108 mode active

no shutdown

interface Ethernet1/13

description FSNETNTX1125-Link2-Nexus2

switchport

switchport mode trunk

channel-group 108 mode active

no shutdown

## FSNETNTX1126 — Po109, vPC 109  (Ethernet1/14 + Ethernet1/15)

! --- NEXUS 1 ---

interface port-channel109

description FSNETNTX1126-vPC109

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 109

no shutdown

interface Ethernet1/14

description FSNETNTX1126-Link1-Nexus1

switchport

switchport mode trunk

channel-group 109 mode active

no shutdown

interface Ethernet1/15

description FSNETNTX1126-Link2-Nexus1

switchport

switchport mode trunk

channel-group 109 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel109

description FSNETNTX1126-vPC109

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 109

no shutdown

interface Ethernet1/14

description FSNETNTX1126-Link1-Nexus2

switchport

switchport mode trunk

channel-group 109 mode active

no shutdown

interface Ethernet1/15

description FSNETNTX1126-Link2-Nexus2

switchport

switchport mode trunk

channel-group 109 mode active

no shutdown

## FSNETNTX1127 — Po110, vPC 110  (Ethernet1/16 + Ethernet1/17)

! --- NEXUS 1 ---

interface port-channel110

description FSNETNTX1127-vPC110

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 110

no shutdown

interface Ethernet1/16

description FSNETNTX1127-Link1-Nexus1

switchport

switchport mode trunk

lacp rate fast

channel-group 110 mode active

no shutdown

interface Ethernet1/17

description FSNETNTX1127-Link2-Nexus1

switchport

switchport mode trunk

lacp rate fast

channel-group 110 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel110

description FSNETNTX1127-vPC110

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 110

no shutdown

interface Ethernet1/16

description FSNETNTX1127-Link1-Nexus2

switchport

switchport mode trunk

lacp rate fast

channel-group 110 mode active

no shutdown

interface Ethernet1/17

description FSNETNTX1127-Link2-Nexus2

switchport

switchport mode trunk

lacp rate fast

channel-group 110 mode active

no shutdown

## FSNETNTX1128 — Po111, vPC 111  (Ethernet1/18 + Ethernet1/19)

! --- NEXUS 1 ---

interface port-channel111

description FSNETNTX1128-vPC111

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 111

no shutdown

interface Ethernet1/18

description FSNETNTX1128-Link1-Nexus1

switchport

switchport mode trunk

lacp rate fast

channel-group 111 mode active

no shutdown

interface Ethernet1/19

description FSNETNTX1128-Link2-Nexus1

switchport

switchport mode trunk

lacp rate fast

channel-group 111 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel111

description FSNETNTX1128-vPC111

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 111

no shutdown

interface Ethernet1/18

description FSNETNTX1128-Link1-Nexus2

switchport

switchport mode trunk

lacp rate fast

channel-group 111 mode active

no shutdown

interface Ethernet1/19

description FSNETNTX1128-Link2-Nexus2

switchport

switchport mode trunk

lacp rate fast

channel-group 111 mode active

no shutdown

## SERVER-SW-9 — Po112, vPC 112  (Ethernet1/20 + Ethernet1/21)

! --- NEXUS 1 ---

interface port-channel112

description SERVER-SW-9-vPC112

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 112

no shutdown

interface Ethernet1/20

description SERVER-SW-9-Link1-Nexus1

switchport

switchport mode trunk

channel-group 112 mode active

no shutdown

interface Ethernet1/21

description SERVER-SW-9-Link2-Nexus1

switchport

switchport mode trunk

channel-group 112 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel112

description SERVER-SW-9-vPC112

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 112

no shutdown

interface Ethernet1/20

description SERVER-SW-9-Link1-Nexus2

switchport

switchport mode trunk

channel-group 112 mode active

no shutdown

interface Ethernet1/21

description SERVER-SW-9-Link2-Nexus2

switchport

switchport mode trunk

channel-group 112 mode active

no shutdown

## SERVER-SW-10 — Po113, vPC 113  (Ethernet1/22 + Ethernet1/23)

! --- NEXUS 1 ---

interface port-channel113

description SERVER-SW-10-vPC113

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 113

no shutdown

interface Ethernet1/22

description SERVER-SW-10-Link1-Nexus1

switchport

switchport mode trunk

channel-group 113 mode active

no shutdown

interface Ethernet1/23

description SERVER-SW-10-Link2-Nexus1

switchport

switchport mode trunk

channel-group 113 mode active

no shutdown

! --- NEXUS 2 ---

interface port-channel113

description SERVER-SW-10-vPC113

switchport

switchport mode trunk

switchport trunk allowed vlan 1-39,45

vpc 113

no shutdown

interface Ethernet1/22

description SERVER-SW-10-Link1-Nexus2

switchport

switchport mode trunk

channel-group 113 mode active

no shutdown

interface Ethernet1/23

description SERVER-SW-10-Link2-Nexus2

switchport

switchport mode trunk

channel-group 113 mode active

no shutdown

# Section 7 — SVI Routing + HSRP (All 31 VLANs)

INFO  Nexus 1 = HSRP active (priority 110 + preempt) for all VLANs. Nexus 2 = standby (priority 100). HSRP virtual IP (.1) is what all downstream devices use as their default gateway. With peer-gateway enabled, both Nexus switches can forward regardless of HSRP state.

WARN  HSRP virtual IPs (.1 per subnet) replace the old CBS SVI IPs (.1 per subnet) — the gateway address hosts use is unchanged. Only the physical Nexus IPs (.2 and .3) are new.

CRITICAL  Update Fortigate and Palo Alto interface gateway config to use .1 virtual IPs (which is what the CBS had). No change needed on hosts — same gateway.

## NEXUS 1 — All SVIs (HSRP Active)

! ============================================================

! SVIs + HSRP — NEXUS 1 (Active, priority 110, preempt)

! Physical IP: .2 | HSRP Virtual: .1

! ============================================================

interface Vlan1

ip address 172.27.8.30 255.255.255.0

hsrp version 2

hsrp 1

ip 172.27.8.1

priority 110

preempt

no shutdown

interface Vlan10

ip address 172.27.10.2 255.255.255.0

hsrp version 2

hsrp 10

ip 172.27.10.1

priority 110

preempt

no shutdown

interface Vlan11

ip address 172.27.11.2 255.255.255.0

hsrp version 2

hsrp 11

ip 172.27.11.1

priority 110

preempt

no shutdown

interface Vlan12

ip address 172.27.12.2 255.255.255.0

hsrp version 2

hsrp 12

ip 172.27.12.1

priority 110

preempt

no shutdown

interface Vlan13

ip address 172.27.13.2 255.255.255.0

ipv6 address 2401:8800:c13:2::85/126

hsrp version 2

hsrp 13

ip 172.27.13.1

priority 110

preempt

no shutdown

interface Vlan14

ip address 172.27.14.2 255.255.255.0

hsrp version 2

hsrp 14

ip 172.27.14.1

priority 110

preempt

no shutdown

interface Vlan15

ip address 172.27.15.2 255.255.255.0

hsrp version 2

hsrp 15

ip 172.27.15.1

priority 110

preempt

no shutdown

interface Vlan16

ip address 172.27.16.2 255.255.255.0

hsrp version 2

hsrp 16

ip 172.27.16.1

priority 110

preempt

no shutdown

interface Vlan17

ip address 172.27.17.2 255.255.255.0

hsrp version 2

hsrp 17

ip 172.27.17.1

priority 110

preempt

no shutdown

interface Vlan18

ip address 172.27.18.2 255.255.255.0

hsrp version 2

hsrp 18

ip 172.27.18.1

priority 110

preempt

no shutdown

interface Vlan19

ip address 172.27.19.2 255.255.255.0

hsrp version 2

hsrp 19

ip 172.27.19.1

priority 110

preempt

no shutdown

interface Vlan20

ip address 172.27.20.2 255.255.255.0

hsrp version 2

hsrp 20

ip 172.27.20.1

priority 110

preempt

no shutdown

interface Vlan21

ip address 172.27.21.2 255.255.255.0

hsrp version 2

hsrp 21

ip 172.27.21.1

priority 110

preempt

no shutdown

interface Vlan22

ip address 172.27.22.2 255.255.255.0

hsrp version 2

hsrp 22

ip 172.27.22.1

priority 110

preempt

no shutdown

interface Vlan23

ip address 172.27.23.2 255.255.255.0

ipv6 enable

hsrp version 2

hsrp 23

ip 172.27.23.1

priority 110

preempt

no shutdown

interface Vlan24

ip address 172.27.24.2 255.255.255.0

hsrp version 2

hsrp 24

ip 172.27.24.1

priority 110

preempt

no shutdown

interface Vlan25

ip address 172.27.25.2 255.255.255.0

hsrp version 2

hsrp 25

ip 172.27.25.1

priority 110

preempt

no shutdown

interface Vlan26

ip address 172.27.26.2 255.255.255.0

hsrp version 2

hsrp 26

ip 172.27.26.1

priority 110

preempt

no shutdown

interface Vlan27

ip address 172.27.27.2 255.255.255.0

ipv6 enable

hsrp version 2

hsrp 27

ip 172.27.27.1

priority 110

preempt

no shutdown

interface Vlan28

ip address 172.27.28.2 255.255.255.0

hsrp version 2

hsrp 28

ip 172.27.28.1

priority 110

preempt

no shutdown

interface Vlan29

ip address 172.27.29.2 255.255.255.0

hsrp version 2

hsrp 29

ip 172.27.29.1

priority 110

preempt

no shutdown

interface Vlan30

ip address 172.27.30.2 255.255.255.0

hsrp version 2

hsrp 30

ip 172.27.30.1

priority 110

preempt

no shutdown

interface Vlan31

ip address 172.27.31.2 255.255.255.0

hsrp version 2

hsrp 31

ip 172.27.31.1

priority 110

preempt

no shutdown

interface Vlan32

ip address 172.27.32.2 255.255.255.0

hsrp version 2

hsrp 32

ip 172.27.32.1

priority 110

preempt

no shutdown

interface Vlan33

ip address 172.27.33.2 255.255.255.0

hsrp version 2

hsrp 33

ip 172.27.33.1

priority 110

preempt

no shutdown

interface Vlan34

ip address 172.27.34.2 255.255.255.0

hsrp version 2

hsrp 34

ip 172.27.34.1

priority 110

preempt

no shutdown

interface Vlan35

ip address 172.27.35.2 255.255.255.0

hsrp version 2

hsrp 35

ip 172.27.35.1

priority 110

preempt

no shutdown

interface Vlan36

ip address 172.27.36.2 255.255.255.0

hsrp version 2

hsrp 36

ip 172.27.36.1

priority 110

preempt

no shutdown

interface Vlan37

ip address 172.27.37.2 255.255.255.0

ipv6 address 2401:8800:c13:2::91/124

hsrp version 2

hsrp 37

ip 172.27.37.1

priority 110

preempt

no shutdown

interface Vlan38

ip address 172.27.38.2 255.255.255.0

hsrp version 2

hsrp 38

ip 172.27.38.1

priority 110

preempt

no shutdown

interface Vlan39

ip address 172.27.39.2 255.255.255.0

hsrp version 2

hsrp 39

ip 172.27.39.1

priority 110

preempt

no shutdown

! VLAN 45 — L2 only, no SVI

ip default-gateway 172.27.8.1

## NEXUS 2 — All SVIs (HSRP Standby)

! ============================================================

! SVIs + HSRP — NEXUS 2 (Standby, priority 100)

! Physical IP: .3 | HSRP Virtual: .1 (same as Nexus1)

! ============================================================

interface Vlan1

ip address 172.27.8.31 255.255.255.0

hsrp version 2

hsrp 1

ip 172.27.8.1

priority 100

no shutdown

interface Vlan10

ip address 172.27.10.3 255.255.255.0

hsrp version 2

hsrp 10

ip 172.27.10.1

priority 100

no shutdown

interface Vlan11

ip address 172.27.11.3 255.255.255.0

hsrp version 2

hsrp 11

ip 172.27.11.1

priority 100

no shutdown

interface Vlan12

ip address 172.27.12.3 255.255.255.0

hsrp version 2

hsrp 12

ip 172.27.12.1

priority 100

no shutdown

interface Vlan13

ip address 172.27.13.3 255.255.255.0

ipv6 address 2401:8800:c13:2::85/126

hsrp version 2

hsrp 13

ip 172.27.13.1

priority 100

no shutdown

interface Vlan14

ip address 172.27.14.3 255.255.255.0

hsrp version 2

hsrp 14

ip 172.27.14.1

priority 100

no shutdown

interface Vlan15

ip address 172.27.15.3 255.255.255.0

hsrp version 2

hsrp 15

ip 172.27.15.1

priority 100

no shutdown

interface Vlan16

ip address 172.27.16.3 255.255.255.0

hsrp version 2

hsrp 16

ip 172.27.16.1

priority 100

no shutdown

interface Vlan17

ip address 172.27.17.3 255.255.255.0

hsrp version 2

hsrp 17

ip 172.27.17.1

priority 100

no shutdown

interface Vlan18

ip address 172.27.18.3 255.255.255.0

hsrp version 2

hsrp 18

ip 172.27.18.1

priority 100

no shutdown

interface Vlan19

ip address 172.27.19.3 255.255.255.0

hsrp version 2

hsrp 19

ip 172.27.19.1

priority 100

no shutdown

interface Vlan20

ip address 172.27.20.3 255.255.255.0

hsrp version 2

hsrp 20

ip 172.27.20.1

priority 100

no shutdown

interface Vlan21

ip address 172.27.21.3 255.255.255.0

hsrp version 2

hsrp 21

ip 172.27.21.1

priority 100

no shutdown

interface Vlan22

ip address 172.27.22.3 255.255.255.0

hsrp version 2

hsrp 22

ip 172.27.22.1

priority 100

no shutdown

interface Vlan23

ip address 172.27.23.3 255.255.255.0

ipv6 enable

hsrp version 2

hsrp 23

ip 172.27.23.1

priority 100

no shutdown

interface Vlan24

ip address 172.27.24.3 255.255.255.0

hsrp version 2

hsrp 24

ip 172.27.24.1

priority 100

no shutdown

interface Vlan25

ip address 172.27.25.3 255.255.255.0

hsrp version 2

hsrp 25

ip 172.27.25.1

priority 100

no shutdown

interface Vlan26

ip address 172.27.26.3 255.255.255.0

hsrp version 2

hsrp 26

ip 172.27.26.1

priority 100

no shutdown

interface Vlan27

ip address 172.27.27.3 255.255.255.0

ipv6 enable

hsrp version 2

hsrp 27

ip 172.27.27.1

priority 100

no shutdown

interface Vlan28

ip address 172.27.28.3 255.255.255.0

hsrp version 2

hsrp 28

ip 172.27.28.1

priority 100

no shutdown

interface Vlan29

ip address 172.27.29.3 255.255.255.0

hsrp version 2

hsrp 29

ip 172.27.29.1

priority 100

no shutdown

interface Vlan30

ip address 172.27.30.3 255.255.255.0

hsrp version 2

hsrp 30

ip 172.27.30.1

priority 100

no shutdown

interface Vlan31

ip address 172.27.31.3 255.255.255.0

hsrp version 2

hsrp 31

ip 172.27.31.1

priority 100

no shutdown

interface Vlan32

ip address 172.27.32.3 255.255.255.0

hsrp version 2

hsrp 32

ip 172.27.32.1

priority 100

no shutdown

interface Vlan33

ip address 172.27.33.3 255.255.255.0

hsrp version 2

hsrp 33

ip 172.27.33.1

priority 100

no shutdown

interface Vlan34

ip address 172.27.34.3 255.255.255.0

hsrp version 2

hsrp 34

ip 172.27.34.1

priority 100

no shutdown

interface Vlan35

ip address 172.27.35.3 255.255.255.0

hsrp version 2

hsrp 35

ip 172.27.35.1

priority 100

no shutdown

interface Vlan36

ip address 172.27.36.3 255.255.255.0

hsrp version 2

hsrp 36

ip 172.27.36.1

priority 100

no shutdown

interface Vlan37

ip address 172.27.37.3 255.255.255.0

ipv6 address 2401:8800:c13:2::91/124

hsrp version 2

hsrp 37

ip 172.27.37.1

priority 100

no shutdown

interface Vlan38

ip address 172.27.38.3 255.255.255.0

hsrp version 2

hsrp 38

ip 172.27.38.1

priority 100

no shutdown

interface Vlan39

ip address 172.27.39.3 255.255.255.0

hsrp version 2

hsrp 39

ip 172.27.39.1

priority 100

no shutdown

! VLAN 45 — L2 only, no SVI

ip default-gateway 172.27.8.1

# Section 8 — Loopback Interfaces

## NEXUS 1

interface loopback0

description NTP-SNMP-Source-Nexus1

ip address 172.27.7.1 255.255.255.252

no shutdown

interface loopback1

description Replicated-CBS

ip address 172.27.7.5 255.255.255.252

no shutdown

## NEXUS 2

interface loopback0

description NTP-SNMP-Source-Nexus2

ip address 172.27.7.2 255.255.255.252

no shutdown

interface loopback1

description Replicated-CBS

ip address 172.27.7.6 255.255.255.252

no shutdown

# Section 9 — Management Plane (Both Switches)

## NTP

ntp server 172.27.13.11 use-vrf default

ntp server 172.27.13.12 use-vrf default

ntp source-interface loopback0

clock timezone IST 5 30

## Syslog

logging server 172.27.13.13

logging server 172.27.13.14

logging server 172.27.13.29

logging level default 6

logging console 2

logging timestamp milliseconds

logging source-interface loopback0

## SNMP

snmp-server community FSNET ro

snmp-server host 172.27.13.14 version 2c FSNET

snmp-server host 172.27.13.19 version 2c FSNET

snmp-server host 172.27.13.191 version 2c FSNET

snmp-server location "Netmagic Datacenter"

snmp-server contact "Nikhil Kadam"

## Users, SSH & Banner

CRITICAL  Replace <ENCRYPTED> with actual encrypted hashes. Never store plaintext passwords in config.

username cnfmgr    password 5 <ENCRYPTED> role network-admin

username fsnetrw   password 5 <ENCRYPTED> role network-admin

username nikhil.k  password 5 <ENCRYPTED> role network-admin

username ajinkya.b password 5 <ENCRYPTED> role network-admin

feature ssh

ssh key rsa 2048

no feature telnet

line console

exec-timeout 5

line vty

exec-timeout 5

transport input ssh

banner motd #

AUTHORIZED ACCESS ONLY

DISCONNECT ASAP IF YOU ARE NOT AUTHORISED

ACCESS MONITORED AND RECORDED

#

# Section 10 — Post-Cutover Verification

INFO  Run in order. vPC must be healthy before connecting any downstream device.

! Step 1 — vPC health (run FIRST on both switches)

show vpc

show vpc brief

show vpc peer-keepalive

show vpc consistency-parameters global

show vpc role

! Step 2 — Port-channels

show port-channel summary

show lacp neighbor

show lacp counters

! Step 3 — STP root confirmation

show spanning-tree summary

show spanning-tree vlan 1 detail

! Step 4 — SVIs and HSRP

show ip interface brief

show hsrp brief

show ip route

! Step 5 — VLAN membership

show vlan brief

! Step 6 — NTP

show ntp peer-status

show clock

! Step 7 — Connectivity

ping 172.27.8.31 source-interface Vlan1

ping 172.27.13.13 source-interface Vlan13

ping 172.27.13.11 source-interface loopback0

! Step 8 — LLDP neighbors

show lldp neighbors

show lldp neighbors detail

! Step 9 — Transceiver health

show interface transceiver

! Step 10 — Error counters baseline

show interface counters errors

Netmagic Nexus vPC Config v2  |  Draft  |  Hassan  |  E1/45-46=FGCore, E1/47-48= Keepalive, E1/49-50=ISL Po50, E1/4-23=Servers Po104-113  |  Verify before applying