Table of Contents

Cisco Catalyst C9300 Switch Hardening Report	2

1. Configuration Overview	2

1.1 General Configuration	2

1.2 Line and Access Configuration	2

1.3 Secure Management Protocols	2

1.4 Logging and Monitoring	3

1.5 Authentication and Encryption	3

1.6 Banners	3

2. Known Vulnerability Mitigation	4

2.1 SSL Certificate Cannot Be Trusted & SSL Self-Signed Certificate	4

2.2 OpenSSH < 9.3p2 Vulnerability	4

2.3 OpenSSH < 9.6 Multiple Vulnerabilities & SSH Terrapin Prefix Truncation Weakness (CVE-2023-48795)	4

2.4 Web Server Uses Basic Authentication Without HTTPS	5

2.5 Web Server Allows Password Auto-Completion	5

2.6 CVE-1999-0524 (ICMP Information Disclosure)	5

2.7 OpenSSH Versions < 7.4, < 7.5, < 7.6, < 7.8, < 8.0, 6.2 < 8.8	5

# Cisco Catalyst C9300 Switch Hardening Report

Date: March 21, 2025
Prepared by: Hassan Rhani.
Device: Cisco Catalyst C9300 Switch
Objective: To secure and harden the Cisco C9300 switch by applying best practices and mitigating identified vulnerabilities using standards CIS Benchmark.

## 1. Configuration Overview

The following configurations have been applied to harden the Cisco C9300 switch. These settings enhance security by restricting unauthorized access, disabling unnecessary services, and implementing secure management practices.

### 1.1 General Configuration

ip domain-name [domain-name]

ip name-server [dns-server-ip]

ntp server [ntp-server-ip]

timezone IST +5 30

crypto key generate rsa modulus 2048

### 1.2 Line and Access Configuration

line aux 0

no exec

transport input none

transport output none

exec-timeout 0 1

no password

line console 0

exec-timeout 10

line vty 0 31

exec-timeout 10

### 1.3 Secure Management Protocols

no ip http server

no ip http secure-server

no ip http authentication local

ip ssh time-out 120

ip ssh authentication-retries 5

ip ssh version 2

no ip bootp server

service tcp-keepalives-in

service tcp-keepalives-out

no service pad

### 1.4 Logging and Monitoring

archive

log config

logging enable

logging buffered 64000

logging console critical

service timestamps debug datetime localtime show-timezone

service timestamps log datetime msec localtime show-timezone

logging source-interface vlan [vlan-id]

login on-success log

login on-failure log

### 1.5 Authentication and Encryption

enable secret 0 P@ssw0rd@!@#

service password-encryption

username admin privilege 15 secret 9 P2ssw0rd@!@#

### 1.6 Banners

banner exec ^CCCC

W A R N I N G

**********************************************************************************************************************************************

* This system is the property of "". It is for authorized use only.                                                                  *

* Unauthorized or improper use of this system may result in administrative disciplinary action and/or civil charges/criminal penalties. *

* By logging in this system you indicate your awareness and consent to these terms of use.                                            *

* If you agree to the terms stated in this warning.                                                                                   *

**********************************************************************************************************************************************

^C

banner login ^C

*************************************************************************

* This Device is the Property of "COMPANY_NAME"                             *

* Unauthorised access is strictly prohibited                            *

*************************************************************************

^C

banner motd ^C

**********************************************************************************************************************************************

* This system is the property of "". It is for authorized use only.                                                                  *

* Unauthorized or improper use of this system may result in administrative disciplinary action and/or civil charges/criminal penalties. *

* By logging in this system you indicate your awareness and consent to these terms of use.                                            *

* If you agree to the terms stated in this warning.                                                                                   *

**********************************************************************************************************************************************

^C

## 2. Known Vulnerability Mitigation

The following vulnerabilities were identified and mitigated as part of the hardening process. Explanations and applied fixes are provided below.

### 2.1 SSL Certificate Cannot Be Trusted & SSL Self-Signed Certificate

- Issue: Use of a self-signed certificate for HTTPS access.

- Mitigation:

no ip http secure-server

- Explanation:
The HTTPS server has been disabled as the device is managed solely via CLI, and GUI/HTTPS access is not required. Disabling the HTTPS server prevents TLS/SSL handshakes, eliminating the need for a trusted CA-signed certificate. While a self-signed certificate remains in the configuration, it is not used when HTTPS is disabled.
For CA-signed certificate installation, refer to:
Cisco Guide: Configure CA-Signed Certificates

### 2.2 OpenSSH < 9.3p2 Vulnerability

- Issue: Vulnerable ciphers in OpenSSH implementation.

- Mitigation:

no ip ssh server algorithm encryption chacha20-poly1305@openssh.com

no ip ssh client algorithm encryption chacha20-poly1305@openssh.com

no ip ssh server algorithm mac hmac-sha2-256-etm@openssh.com

no ip ssh client algorithm mac hmac-sha2-256-etm@openssh.com

no ip ssh server algorithm mac hmac-sha2-512-etm@openssh.com

no ip ssh client algorithm mac hmac-sha2-512-etm@openssh.com

- Explanation:
Removed affected encryption and MAC algorithms to mitigate vulnerabilities in OpenSSH versions prior to 9.3p2.

### 2.3 OpenSSH < 9.6 Multiple Vulnerabilities & SSH Terrapin Prefix Truncation Weakness (CVE-2023-48795)

- Issue: Vulnerable MAC algorithms susceptible to prefix truncation attacks.

- Mitigation:

ip ssh client algorithm mac hmac-sha2-256 hmac-sha2-512

ip ssh server algorithm mac hmac-sha2-256 hmac-sha2-512

no ip ssh server algorithm encryption chacha20-poly1305@openssh.com

no ip ssh client algorithm encryption chacha20-poly1305@openssh.com

no ip ssh server algorithm mac hmac-sha2-256-etm@openssh.com

no ip ssh client algorithm mac hmac-sha2-256-etm@openssh.com

no ip ssh server algorithm mac hmac-sha2-512-etm@openssh.com

no ip ssh client algorithm mac hmac-sha2-512-etm@openssh.com

- Explanation:
Configured secure MAC algorithms (hmac-sha2-256, hmac-sha2-512) without the vulnerable -etm@openssh.com suffix and removed affected ciphers to address CVE-2023-48795.

### 2.4 Web Server Uses Basic Authentication Without HTTPS

- Issue: HTTP server enabled without HTTPS.

- Mitigation:

no ip http secure-server

- Explanation:
Disabled the HTTP server as it is not required for management, eliminating the risk of unencrypted authentication.

### 2.5 Web Server Allows Password Auto-Completion

- Issue: Potential password exposure via auto-completion.

- Explanation:
Per Cisco bug CSCwi89826, IOS-XE devices are not affected by this vulnerability. No further action is required.

### 2.6 CVE-1999-0524 (ICMP Information Disclosure)

- Issue: ICMP allows netmask and timestamp information from arbitrary hosts.

- Mitigation:

access-list 123 deny icmp any any timestamp-request

access-list 123 deny icmp any any timestamp-reply

access-list 123 permit ip any any

interface [L3-interface]

ip access-group 123 in

- Explanation:
Applied an ACL to deny ICMP timestamp requests and replies, preventing information disclosure. Refer to:
Cisco Community: Fix for CVE-1999-0524

### 2.7 OpenSSH Versions < 7.4, < 7.5, < 7.6, < 7.8, < 8.0, 6.2 < 8.8

- Issue: Potential vulnerabilities in older OpenSSH versions.

- Explanation:
The current software version (IOS-XE 17.12.4) is not affected by these vulnerabilities, as confirmed by:

  - Bug CSCvy91615

  - Cisco Software Checker: Link

## 3. Additional Notes

- Reference Template: Cisco Access Lists Guide

- Software Version: IOS-XE 17.12.4 (verified as secure against listed OpenSSH vulnerabilities).

- Recommendations:

  - Regularly update the switch firmware to address newly discovered vulnerabilities.

  - Review and update passwords periodically to maintain security.